Current version from 2022-07-29

Annex 2 Appendix 2 to the Procurement Conditions

PROPOSED AIRCRAFT TOW VEHICLE (MEDIUM POWER)

TECHNICAL SPECIFICATIONS

| No. | Requirements | Compliance with the requirements (to be completed by the Supplier, indicating a specific value (if requested) or whether the offered product meets or does not meet a specific requirement) |
| --- | --- | --- |
| 1. | General requirements for the tow vehicle |  |
| 1.1 | structure: body, operator cab, chassis, transmission, engine; | YES |
| 1.2. | the tow vehicle will be operated in a fenced-in area of the airport (with an asphalt, concrete or hard soil surface, which in the cold season is treated with ice- and snow-melting agents) in typical A3-C3 environments (according to NATO STANAG 4370 Environmental testing or equivalent); | YES |
| 1.3. | operation: the tow vehicle is operated by a single operator; | YES |
| 1.4. | driving speed: maximum driving speed not less than 25 km/h; | YES specify the exact value in km/h, what is the maximum driving speed: 26 km/h |
| 1.5. | standards: engine emission standard shall be not higher than EURO 3; | YES |
| 1.6. | aircraft tow vehicle’s maximum drawbar pull (DBP) – not less than 70kN; | YES specify the exact value in kN of the tensile strength of the coupling joints: 105 kN |
| 1.7. | operating capacity of the tow vehicle: towing aircraft or cargo dollies with a total mass of not less than 90 tonnes, ensuring a climb/descent of not less than 2% gradient; | YES |
| 1.8. | the capacity of the fuel tank(s) of the tow vehicle shall be such that the aircraft tow vehicle can operate/drive for not less than 8 hours without refuelling; | YES |
| 2. | Requirements for the body of the tow vehicle |  |
| 2.1. | colour: green (RAL6031); | YES |
| 2.2. | the surfaces of the vehicle body shall be covered with anti-corrosion coating resistant to ice- and snow-melting agents and salts; | YES, protection based on normal practice in GSE industry. |
| 2.3. | reflectors and limit marks at the front, sides and rear of the vehicle body; | YES |
| 2.4. | the minimum weight of the tow vehicle shall be not less than 9 tonnes; | YES specify the exact value in t what is the minimum weight of the tractor:9t For the defined operation we recommend vehicle weight of 11t. |
| 2.5. | front and rear couplings: |  |
| 2.5.1. | coupling device – not less than 3 stages at the front and at the rear, the lowest stage being not less than 30 cm in height measured from the ground; | YES. We can mount the tow hitch as defined by you. specify the exact value: 30 cm |
| 2.5.2. | front and rear couplings shall be equipped with lighting; | YES |
| 2.5.3. | if the operator does not see the couplings from the cab, additional mirrors shall be provided for safe operation. | YES |
| 3. | Requirements for the operator cab of the tow vehicle |  |
| 3.1. | an enclosed operator cab in accordance with IATA AHM 955; | YES |
| 3.2. | the cab is designed for comfortable operation by the operator: |  |
| 3.2.1. | the cab shall be equipped with an air-conditioning system that allows comfortable operation during the warm and cold seasons; | YES |
| 3.2.2. | front and rear windscreen wipers with washing function; | YES |
| 3.2.3. | electrically-heated rear glass; | YES |
| 3.2.4. | sun visor on the front windscreen; | YES |
| 3.2.5. | rear-view mirrors: not less than 2 on the sides on the exterior of the cab, heated, 1 inside the cab; | YES |
| 3.2.6. | illuminated dashboard; | YES |
| 3.2.7. | not less than 2 seats in the cab; | YES |
| 3.2.8. | adjustable driver’s seat; | YES |
| 3.2.9. | not less than 1 orange-coloured beacon light fitted on the cab; | YES |
| 3.2.10. | rubber mats shall be on the floor of the cab. | YES |
| 4. | Requirements for engine, transmission, brakes and chassis |  |
| 4.1. | Engine: |  |
| 4.1.1. | designed to use diesel fuel, standard lubricants and operating fluids; | YES |
| 4.1.2. | engine power – not less than 55 kW; | YES specify the exact value in kW, what is the motor power: 74,4 kW |
| 4.1.3. | shall start without additional equipment or preparation when the air temperature is as low as -10 °C; | YES |
| 4.1.4. | at an air temperature lower than -10 °C, engine start-up shall be facilitated by installed additional heating equipment for the engine; | YES |
| 4.1.5. | in case of an engaged gear, engine-start shall be blocked. | YES |
| 4.2. | Transmission: |  |
| 4.2.1. | fully automatic; | YES |
| 4.2.2. | the gears must switch gently without changes in the pulling force; | YES |
| 4.3. | Brakes: |  |
| 4.3.1. | service brakes (pneumatic or hydraulic, disc) for all four wheels; | YES, wet lamella disc brakes |
| 4.3.2. | the parking brake shall reliably protect the tow vehicle from moving, even when located on a downhill; | YES |
| 4.3.3. | the operator’s cab shall be equipped with a light indicating the engagement of parking brakes; | YES |
| 4.3.4. | in case of malfunction, the emergency braking system shall be provided with a release function. | YES |
| 4.4. | Chassis: |  |
| 4.4.1. | all-wheel drive, steering (4WD) with uniform weight distribution on both axles of the chassis, which ensures the maximum pulling force of the tow vehicle; | YES |
| 4.4.2. | pneumatic tyres; | YES |
| 4.4.3. | equipped with power steering. | YES |
| 5. | Requirements for electrical appliances and wiring |  |
| 5.1. | electrical system 12V or 24V; | YES specify the exact value: 24 V |
| 5.2. | light devices: low beam (dipped), high beam (full beam), emergency, side marker, reverse (backup), brake lights; | YES |
| 5.3. | additional operating lights (LEDs) shall be installed on the cab of the tow vehicle and at its rear for safe operation near the aircraft during hours of darkness; | YES |
| 5.4. | vehicle horn (signal); | YES |
| 5.5. | reverse (backup) audible signal; | YES |
| 5.6. | the technical compartment shall be equipped with a battery disconnect switch; | YES |
| 5.7. | the dashboard shall include: |  |
| 5.7.1. | counter for motor hours of service; | YES |
| 5.7.2. | fuel level indicator; | YES |
| 5.7.3. | coolant temperature indicator; | YES |
| 5.7.4. | light and sound alarm on the dashboard; | YES |
| 5.7.5. | additional light indicators are installed on the dashboard to inform about insufficient engine oil quantity, low engine oil pressure, lack of coolant; | YES |
| 5.7.6. | all electrical devices located outside the cab shall be resistant to atmospheric effects. | YES |
| 6. | Warranty requirements |  |
| 6.1. | a guarantee of not less than 2 years shall be provided; | YES specify the exact value: 2 years |
| 6.2. | the guarantee shall take effect from the date of transfer to the buyer; | YES |
| 6.3. | the periodic technical services specified in the manufacturer’s instructions shall be performed at the supplier’s expense during the warranty period at the following address: Lakūnų g. 3, Šiauliai; | YES |
| 6.4. | 3.4. all malfunctions occurring during the warranty period not due to the fault of the operator shall be resolved at the supplier’s expense within a maximum of 30 calendar days. | YES |
| 7. | Additional information: |  |
| 7.1. | the tow vehicle must be new, unused, in serial production, in good working order and ready for operation; | YES |
| 7.2. | all inscriptions and markings relating to the operation of the tow vehicle shall be in Lithuanian or English; | YES |
| 7.3. | staff training for 10 people: |  |
| 7.3.1. | training shall be provided for personnel operating the tow vehicle, performing day-to-day maintenance and post-warranty service. Certificates shall be issued after the completion of the training; | YES |
| 7.3.2. | the training shall be organised at: Lakūnų g. 3, Šiauliai; | YES |
| 7.4. | equipment set: |  |
| 7.4.1. | a catalogue of spare parts with part identification numbers in English on a digital medium as well as printed; | YES |
| 7.4.2. | tow vehicle driver/operator manual and maintenance documentation (in English and Lithuanian) on a digital medium as well as printed; | YES |
| 7.4.3. | in the tow vehicle, there shall be easy access to not less than one 5-kg fire extinguisher and first-aid kit; | YES |
| 7.4.4. | the rear part shall be provided with a space for affixing the number plate and illumination of the number plate shall be provided; | YES |
| 7.4.5. | a spare wheel suitable for the chassis, a position for affixing the wheel may be provided; | YES |
| 7.4.6. | tow vehicle civil liability insurance policy valid for 12 months from the date of signature of the act of transfer-acceptance of the tow vehicle. | YES, based on liability insurance. Liability limit 60 mio. EUR per insured event and 60 mio EUR per insurance year. |