Annex 1

to the Contract

No.

dated __ _____ 2024

TECHNICAL SPECIFICATION FOR UNARMOURED (LIGHTLY ARMOURED) OFF-ROAD VEHICLE

|  | Requirements | Specifications offered by the Seller |
| --- | --- | --- |
| 1. GENERAL PART |  |  |
| 1.1. | 1.1. A military tactical level unarmoured ((lightly armoured, protection level 1 according to STANAG 4569 or protection level 2 according to STANAG 4569 or equivalent)) off-road vehicle (hereafter referred to as off-road vehicle) is intended for the transport of soldiers and cargo as well as for the performance of combat tasks in Lithuania and in international operations abroad. The off-road vehicle must be new and unused. | YES |
| 1.2. 1.2.1. 1.2.2. | The off-road vehicle is expected to be in service: In climate zones A2, A3, B1, B2, C0 and C1 according to STANAG 4370 or equivalent; On a paved or gravel road, on dry paths in fields, on hard-surfaced agri-cultivated ploughs and on agri-cultivated ploughs without paths: Type 1 (sandy loam and sandy soils), Type 2 (sandy and clay soils) and Type 3 (clayey and silty clay soils) in accordance with STANAG 2592 or equivalent. | YES YES |
| 1.3. | The threaded connections and scales of the off-road vehicle’s measuring instruments shall be in the metric system of measurement. | YES |
| 1.4. | The off-road vehicle is designed for the right side of the road and the steering wheel is on the left side. | YES |
| 1.5. | At the maximum gross vehicle weight, the engine power must be capable of a minimum speed of 90 km/h on hard-surfaced roads. | 130 KM/H |
| 1.6. | The off-road vehicle must comply with the technical requirements set out in Order No. 2B-290 of 29 July 2008 of the Head of the State Road Transport Inspectorate under the Ministry of Transport No. 2B-290 “On the Technical Requirements for Motor Vehicles and their Trailers” (current version). | YES |
| 2. PARAMETERS |  |  |
| 2.1. OFF-ROAD WEHICLE |  |  |
| 2.1.1. | Wheelbase (distance between the axles of the wheels) not less than 2850 mm. | 3 100 MM |
| 2.1.2. | Overall length not exceeding 5200 mm. | 5 100 MM |
| 2.1.3. | Maximum width (excluding rear-view mirrors) not exceeding 2000 mm. | 1 800 MM |
| 2.1.4. | Height not more than 2300 mm (with turret to fit a Boeing CH-47 Chinook). | 1 870 MM |
| 2.1.5. | Ground clearance not less than 220 mm, measured as follows: | 300 MM |
| 3. PULLING/TOWING |  |  |
| 3.1. OFF-ROAD VEHICLE: |  |  |
| 3.1.1. | Must have towing loops at the front and rear. | YES |
| 3.1.2. | Must have a front towing coupling (pin). | YES |
| 3.1.3. | Must have a Ringfeder Ruwg K2D removable rear towing hitch or equivalent. | YES |
| 3.1.4. | Must have a detachable towing hook. | YES |
| 3.1.5. | Must have a draw pulley of at least 80 kN. | ≥80 kN |
| 3.1.6. | It shall have a front electric winch with a towing capacity of at least 40 kN and a protective cover, with a length of synthetic rope of at least 25 m. | ≥ 40 kN ≥ 25 m |
| 3.1.7. | The electric winch must be operated by remote control and/or wired remote control. | Wired remote control |
| 4. TRANSPOTABILITY |  |  |
| 4.1. | The off-road vehicle shall be suitable for transport by rail, ship and allied aircraft and shall comply with STANAG 4062 or equivalent. | YES |
| 4.2. | The off-road vehicle shall be equipped with loading and transport anchorages according to STANAG 4478, STANAG 4062, STANAG 7213 or equivalent. | YES |
| 5. ARMOUR |  |  |
| 5.1. | Not less than one door with windows in accordance with STANAG 4569 level 1 or equivalent. | 4 |
| 5.2. | Front windows in accordance with STANAG 4569 level 1 or equivalent. | YES |
| 5.3. | Bottom protection according to STANAG 4569 level 1 or equivalent. | YES |
| 6. BODY |  |  |
| 6.1. | Safety hoops inside to protect the crew in the event of a rollover. | YES |
| 6.2. | Removable doors (without special tools). | YES |
| 6.3. | Interior and exterior attachment points for rucksacks and other items (modelling system) (location to be agreed with the customer at the time of production). | YES |
| 6.4. | Safety hoops at the front and rear. | YES |
| 6.5. | Chassis protection against stones and stumps. | YES |
| 6.6. | The outer part shall provide two places for antenna mounting brackets (WHF and HF type), the location to be agreed with the customer at the time of manufacture. | YES |
| 6.7. | The roof shall be equipped with a strut and a swivel ring carriage for mounting 7,62 mm FN MAG, 50 Cal BMG, 7,62 mm Minigun Aero M134D machine guns (Vinghøg 7,62 mm Buffermount or equivalent) and GMG automatic grenade launchers. The strength/resistance of all structures shall be sufficient to withstand the recoil force of this machine gun. The angle of fire of the machine gun shall be 360°. | YES |
| 6.8. | The machine gun’s swivel ring can be recessed inside the off-road vehicle due to height restrictions. | YES |
| 6.9. | The operator of the machine gun must be able to disconnect the mechanical handle. | YES |
| 6.10. | Ammunition magazines shall be placed on the sides of the turret (location to be agreed with the customer at the time of manufacture). | YES |
| 6.11. | Convenient location for the turret machine gun spare barrel with quick-locking system (location and locking system to be agreed with the customer at the time of manufacture). | YES |
| 6.12. | A machine gun mount of at least 5,56 mm is provided in the front passenger seat. | YES |
| 6.13. | The boot sides open and can be removed without special tools. | YES |
| 6.14. | The off-road vehicle must have a mounting system for transporting the motorcycle (minimum 200 KG). | ≥ 200 kg |
| 6.15. | Tent for tower, roof, electric crane and boot. | YES |
| 6.16. | The body (exterior and interior) must have the maximum number of fasteners for the modular system. | YES |
| 6.17. | Fasteners for two fuel canisters (20l) (location and design to be agreed with the customer at the time of manufacture). | YES |
| 6.18. | Reinforced bonnet with walk-on capability for a soldier no lighter than 150 kg. | ≥ 150 kg |
| 6.19. | Rear rails must have folding-removable shelves. | YES |
| 6.20. | The off-road vehicle must have a normal-sized spare wheel and a designated place for fixing it. | YES |
| 6.21. | The off-road vehicle must have an additional set of closed unarmoured doors. | YES |
| 7. ENGINE AND TRANSMISSION |  |  |
| 7.1. | Diesel with an output of at least 160 kW. | 160 kW |
| 7.2. | Torque of at least 500 Nm. | 500 Nm |
| 7.3. | The engine cooling system must be filled with a coolant that will not freeze to -32°C. | YES |
| 7.4. | 4x4 formula, all-wheel drive. | YES |
| 7.5. | Gearbox. | YES |
| 7.6. | The off-road vehicle must have a downshift gear. | YES |
| 7.7. | 100% differential lock, electrically selectable on the move, gearbox, front and rear axles. | YES |
| 7.8. | Road mobility with a single drive axle or permanent all-wheel drive. | Permanent all-wheel drive |
| 7.9. | Crossing a water obstacle not shallower than 650 mm. | 860 MM |
| 8. CAB |  |  |
| 8.1. | Spaces for stowage and attachment of crew ammunition magazines (location and design to be agreed with the customer at the time of manufacture). | YES |
| 8.2. | Holders for individual and collective weapons (location and design to be agreed with the customer at the time of manufacture). | YES |
| 8.3. | Storage net in the passenger leg area (location and design to be agreed with the customer at the time of manufacture). | YES |
| 8.4. | All vehicle seats are ergonomically adjustable. | YES |
| 8.5. | Shot direction detection system (Pilar, Boomerang, Acusonic or equivalent). | PILAR V |
| 8.6. | Must be able to install a thermal imaging system (eye) (TacFLIR 240 or equivalent). | YES |
| 8.7. | Signal attenuator (communication, C-IED) (Netline or equivalent). | NETLINE |
| 8.8. | Space is provided for a tablet, computer BLUE FORCE TRACKER, ATAK (location and design to be agreed with the customer during production). | YES |
| 8.9. | Fluid warning indicator (engine oil/coolant,). | YES |
| 8.10. | Adjustable dashboard lighting. | YES |
| 8.11. | Adjustable angle and height steering wheel. | YES |
| 8.12. | Information stickers in Lithuanian. | YES |
| 8.13. | Three- or five-point seat belts. | 4 |
| 8.14. | Seats with a headrest. | YES |
| 8.15. | Sun visors for driver and front passenger. | YES |
| 8.16. | Speedometer, rev counter, odometer, odometer, digital speedometer. | YES |
| 8.17. | Anti-scratch coating on the entire floor area. | YES |
| 8.18. | Water drain plug with leak protection. | YES |
| 8.19. | Machine gun operator's seating-parking system (set) (location and design to be agreed with the customer at the time of manufacture). | YES |
| 8.20. | Disc brakes on both axles. | YES |
| 9. WHEELS |  |  |
| 9.1. | Rims not smaller than R16 (rim suitable for use with commercially available tyres), reinforced, off-road (including spare wheel) or equivalent. | R18 POR tyres |
| 9.2. | The off-road vehicle must have a tyre pressure monitoring system (without the driver leaving the cab). | YES |
| 9.3. | The off-road vehicle must have an air compressor for inflating tyres. | YES |
| 9.4. | Run flat tyres; | YES |
| 9.5. | Wheel chain set pewag radial spur SED or equivalent. | RUD PROFI GREIFSTEG |
| 9.6. | Off-road tyres must be designed for off-road, military use. | YES |
| 10. ELECTRICAL EQUIPMENT |  |  |
| 10.1. | Driver Vision Enhancers (Leonardo or equivalent). | LEONARDO DVE WIDES |
| 10.2. | IR lighting (front and rear). | YES |
| 10.3. | Generator 12v and 24v (150 A), protected against dirt. | YES |
| 10.4. | Rechargeable batteries 1 x 12V/92Ah. | YES |
| 10.5. | Electrical socket for trailer 12-socket 24 V according to STANAG 4007 or equivalent. | YES |
| 10.6. | Electric socket for trailer 13-socket 24 V and 7-socket 24 V. | YES |
| 10.7. | External starter connection socket according to STANAG 4074 or equivalent. | YES |
| 10.8. | DC socket 24V/15A and/or 24V/30A. | 24V 15A |
| 10.9. | Voltage converter from 12V/24V to 220V, 1 kW. | YES |
| 10.10. | Additional AGM batteries for communication equipment. | YES |
| 10.11. | Mounted rear-view camera, brightness not less than 400.00 cd/m², complying with IP40/IP69 and connected to a separate screen of not less than 7" HD resolution, protected against dirt (sealed). | AXION |
| 10.12. | Hermetic fog lamps. | YES |
| 10.13. | The headlamps of the off-road vehicle shall be equipped with light masking equipment according to STANAG 4381 or equivalent. | YES |
| 10.14. | Off-road electronic management system with diagnostic system. | YES |
| 10.15. | A switch in the cabin, accessible to the driver, for the control (and complete switch-off) of all the lights of the off-road vehicle in accordance with STANAG 4381 or equivalent. | YES |
| 10.16. | External manual battery switch. | YES |
| 10.17. | Instrument charging sockets in the cab. | YES |
| 11. VENTILATION AND HEATING |  |  |
| 11.1. | The off-road vehicle must be equipped with a cab heating system that maintains the interior temperature of the cab at a minimum of +15°C (when the doors are closed) during winter operation; | ≥ 15°C |
| 11.2. | Shall be an air conditioner to ensure a cabin interior temperature of at least +25°C in hot climate conditions, or a fan/cab vents to ensure comfortable operation in hot weather; | FAN/CAB vents |
| 11.3. | The off-road vehicle must have a self-contained cab and engine heater; | YES |
| 11.4. | The off-road vehicle must have a windscreen heating, cleaning and rinsing system. | YES |
| 12. PAINTING, ANTI-CORROSION COATING |  |  |
| 12.1. | The off-road vehicle shall be painted in military colour in accordance with STANAG 4360 or equivalent. The surface shall be resistant to chemical agents (CARC or equivalent). Colour RAL 6031, matt. The frame, frame parts and wheels must be painted black; | YES |
| 12.2. | Bottom, chassis and body corrosion protection shall be Dinitrol (bituminous base) or equivalent; | Dinitrol |
| 12.3. | The glossy parts of the off-road vehicle are painted matt or black. | MATT/BLACK |
| 13. PACKAGING |  |  |
| 13.1. | Emergency triangle and lamp. | YES |
| 13.2. | Rescue kit. | YES |
| 13.3. | Factory tool kit. | YES |
| 13.4. | Support (wheel) wedges. | YES |
| 13.5. | Off-road vehicle jack. | YES |
| 13.6. | 2 kg fire extinguisher with holder. | YES |
| 13.7. | Vehicle jack base plate. | YES |
| 13.8. | First aid kit. | YES |
| 13.9. | Acoustic or visual differential lock signal. | Visual |
| 13.10. | Radiator and engine crankcase protection. | YES |
| 13.11. | Automatic daytime lights. | YES |
| 13.12. | Fuel preheating and water settling in the fuel system. | YES |
| 13.13. | Headlamp protection grids. | YES |
| 13.14. | Direction indicator lamp protection grids. | YES |
| 13.15. | Rear lamp protection grids. | YES |
| 13.16. | Intake air heating. | YES |
| 13.17. | Cyclonic filter for engine intake air. | YES |
| 13.18. | Spherical rear-view mirrors. | YES |
| 13.19. | External starter cable (6 m). | YES |
| 13.20. | Tyre inflation hose with pressure gauge. | YES |
| 13.21. | Reading lamp in the cab. | YES |
| 13.22. | Equipment for masking all lights and reflective surfaces. | YES |
| 13.23. | Masking nets set (winter, autumn and summer). | YES |
| 13.24. | Larger fuel tank capacity. | YES |
| 13.25. | Constant speed support system. | YES |
| 13.26. | Front towing hook. | YES |
| 13.27. | Trailer coupling device (hook type). | YES |
| 13.28. | Cargo compartment with C-rail and lashing eyelets, including: cargo compartment with C-rail, lashing eyelets. | YES |
| 13.29. | Signal lamp. | YES |
| 13.30. | Smoke screen system front and rear (set). | YES |
| 13.31. | Telescopic lifting mechanism for special surveillance equipment at a height of at least 4 m (location and design to be agreed with the customer at the time of manufacture). | COMROD |
| 13.32. | Vehicle assistance kit (shovel, seat belt knife, etc.). | YES |
| 13.33. | Rucksack attachment nets. | YES |
| 14. MAINTENANCE |  |  |
| 14.1. DOCUMENTATION |  |  |
| 14.1.1. |
| 14.1.1.1. | User manual. | YES |
| 14.1.1.2. | PDF in Lithuanian and English. | YES |
| 14.1.1.3. | 5 printed versions in Lithuanian, plus one printed version in Lithuanian for each off-road vehicle. | YES |
| 14.1.1.4. | Maintenance and repair manual. | YES |
| 14.1.1.5. | PDF and/or interactive format (free 25-year licence with unlimited installation on National Defence System computers) in Lithuanian and/or English. | PDF |
| 14.1.1.6. | 20 printed versions in Lithuanian, plus one printed version in Lithuanian for each off-road vehicle. | YES |
| 14.1.1.7. | Spare parts catalogue in PDF or interactive format (unlimited licence to install on National Defence System computers) in English with serial numbers and/or NSN codes. | PDF |
| 14.1.1.8. | List of the most important parts with part codes in PDF format in English and Lithuanian. | YES |
| 14.1.1.9. | Instructions for preparation for long-term storage (preservation) in Lithuanian in PDF format. | YES |
| 14.1.1.10. | Instructions in Lithuanian and English for the preparation of the off-road vehicle for transport by aircraft, ships, trains (loading/anchoring plan with photos, describing axle weights, off-road vehicle mounting diagrams and attachment point loads, and other information necessary for the preparation of the off-road vehicle for transport). | YES |
| 14.1.2. | All documentation must be submitted for review no later than two months before the delivery of the off-road vehicle. The final version of the documentation shall take into account any recommendations for changes and additions to the documentation made by the purchaser following the review. | YES |
| 14.1.3. | The documentation must be submitted with the first batch of trucks. | YES |
| 14.2. SPECIAL TOOLS |  |  |
| 14.2.1. | The supplier shall provide 2 sets of special tools for the repair of the off-road vehicle Level II. | YES |
| 14.2.2. | The supplier shall provide 1 set of special tools for the partial repair of an off-road vehicle at level III. | YES |
| 14.2.3. | The supplier shall provide 1 set of computerised off-road diagnostics with software. | YES |
| 14.3. PERSONNEL TRAINING |  |  |
| 14.3.1. | The manufacturer must provide an off-road vehicle driver/operator training course (up to 10 participants) in either Lithuanian or English no later than one month after the first off-road vehicle is delivered. The course shall cover Level I repairs (maintenance by the driver/operator). Participants must be certified to train other off-road vehicle drivers/operators and instructors. | YES |
| 14.3.2. | The manufacturer must provide an off-road vehicle repair course (up to 10 participants) in Lithuanian or English no later than one month after the first delivery of the off-road vehicle. The course shall cover level I to II repairs (replacement of assemblies, computer diagnostics, computer troubleshooting and wiring, etc.). Participants must be certified to train other off-road vehicle repairers and instructors. | YES |
| 14.3.3. | All methodological material (presentations) used in the training must be in Lithuanian and must be made available to participants in advance. | YES |
| 14.4. SPARE PARTS |  |  |
| 14.4.1. |
| 14.4.1.1. | a list of the off-road vehicle’s low-wear parts and preventive maintenance for a 3-year period, with prices, serial numbers and/or NSN codes. | YES |
| 14.4.1.2. | a list of spare parts for the off-road vehicle for a period of 3 years. Spare parts availability requirement (likelihood that the spare part is available in the army's warehouse) - 90% of the spare parts must be listed by the supplier with prices, serial numbers and/or NSNs. | YES |
| 14.4.2. | The supplier must guarantee that for a period of 25 years from the date of signing of the transfer of the off-road vehicle, it will be possible to purchase spare parts for the off-road vehicle from at least one supplier in Lithuania. | YES |
| 14.5. REPAIR |  |  |
| 14.5.1. | The supplier must guarantee that, for a period of 25 years from the date of signing of the transfer of the off-road vehicle, it will be possible to obtain the repair of the off-road vehicle from at least one supplier in Lithuania. | YES |
| 14.6. WARRANTY FOR THE OFF-ROAD VEHICLE AND ACCESSORIES |  |  |
| 14.6.1. | The Supplier must provide a detailed plan of how the warranty repairs will be carried out together with the product. | YES |
| 14.6.2. | The warranty is for at least 2 years from the date of signing of the handover and acceptance certificate. | 2 YEARS |
| 14.6.3. | Warranty repairs must be carried out within a maximum of 60 calendar days, with a corresponding extension of the warranty period. | 60 DAYS |
| 14.6.4. | The Supplier shall ensure that all materials supplied and all systems, equipment and components comply with the requirements of this specification until the end of the warranty period. | YES |
| 14.6.5. | During the warranty period, the manufacturer’s materials, spare parts, repair equipment and personnel will be used. | YES |
| 14.6.6. | The supplier may offer a longer warranty for the “powertrain” of the vehicle. | 12 MONTHS |
| QUALITY ASSURANCE |  |  |
| 14.7. | The design (drawings) of the off-road vehicle must be agreed with the buyer before production of the first model starts. | YES |
| 14.8. | The design of the off-road vehicle (the first production) has to be agreed with the buyer before series production can start. The purpose of the design evaluation is to determine whether the off-road vehicle complies with the requirements set out in this Technical Specification. | YES |
| 14.9. | The design of the off-road vehicle cannot be changed without the buyer’s consent. | YES |
| 14.10. | The vehicle cannot be a one-off (experimental) product. | YES |
| 15. DELIVERY |  |  |
| 15.1. | The supplier must deliver the off-road vehicles to the Lithuanian Armed Forces Depot Service Vehicle and Equipment Depot (address: LT-45252, Kaunas, A. Juozapavičiaus pr. 11, Lithuania). | YES |
| 15.2. | Terms of delivery - DDP according to Incoterms 2020. | YES |

| PIRKĖJAS: Gynybos resursų agentūra prie Krašto apsaugos ministerijos Direktorius Sigitas Dzekunskas _________________________ (Parašas) | THE SELLER: Defenture B.V. CEO Mr. Henk van der Sheer _________________________ (Signature) |
| --- | --- |