## TECHNICAL SPECIFICATION FOR

# THE PURCHASE OF MAINTENANCE AND REPAIR SERVICES FOR

# ANTENNA CUFFS OF AIRPORT ACCESS SURVEILLANCE RADARS

# USED BY STATE ENTERPRISE ORO NAVIGACIJA

- ABBREVIATIONS AND CONCEPTS

- The following abbreviations and concepts are used in the technical specification:

Service Provider – a participant, the offer of which shall be acknowledged as the winning following the procedure established under the purchase terms and conditions and with which the Contract shall be concluded.

Purchaser – state enterprise Oro Navigacija.

Contract – a Service Contract signed between the Supplier and the Purchaser.

Services – maintenance and repair services for antenna cuffs of airport access surveillance radars used by state enterprise Oro Navigacija.

Order – a written order of the Services submitted by the Purchaser to the Service Provider on the provision of the Services.

- PURPOSE

2.1. State enterprise Oro Navigacija uses 3 radar systems, each of which is composed of primary radar “Ural” (in Vilnius) or “Morava” (in Kaunas and Palanga) and secondary radar IRS-20MP/S mounted together.

2.2. The radar antenna systems are mounted on the towers. The height of the antenna towers is 18 m (in Vilnius) and 24 m (in Kaunas and Palanga).

2.3. The radar antenna systems at all three locations are protected using S38-75 ESSCO type antenna cuffs.

2.4. The purpose of this purchase is to acquire maintenance and repair services for antenna cuffs of Vilnius, Kaunas and Palanga airport access surveillance radars used by state enterprise Oro Navigacija and indicated under Item 2.3. of this technical specification.

- DESCRIPTION OF THE SERVICES

3. The purchased Services encompass the following groups of services:

  - Maintenance services for antenna cuffs of S38-75 model (in accordance with the documentation of the manufacturer).

  - Cleaning services for antenna cuffs of S38-75 model.

  - Repair services for antenna cuffs of S38-75 model (in cases, when during the maintenance works it is determined that repair works are necessary).

- ORDER AND TERMS OF SERVICE PROVISION

4. General requirements for organisation and performance of the Services:

4.1. Since the environmental impact upon antenna cuffs can vary significantly depending on the local conditions, the regularity of cuff maintenance can be different. Taking into consideration the requirements of the manufacturer of the cuffs (L3 Communications ESSCO) there is stated periodicity of 3 years for performing maintenance works on antenna cuffs of the type S38-75 ESSCO. Therefore, the Purchaser intends to conclude a Contract with the Service Provider for the term of three years, scheduling that every year the maintenance of the antenna cuffs of only one radar operated by the Purchaser shall be performed.

4.2. The Service Provider shall perform all the Services indicated under this technical specification, using the tools and means of his own (except for the water, used for cleaning the antenna cuffs). The Service Provider shall ensure the possession of sufficient means and equipment necessary for execution of the Contract.

4.3. While providing the services the documentation of the manufacturer of S38-75 model antenna cuffs (L-3 ESSCO TM09-15 INSTALLATION, OPERATION & MAINTENANCE MANUAL MODEL S38-75 RADOME) shall be followed by the Service Provider.

4.4. The antenna cuff maintenance services shall include at least:

4.4.1. The maintenance of all antenna cuff panels.

4.4.2. The maintenance of all cuff panels bolts.

4.4.3. Inspection of the torques of all anchor bolts of the cuff (as well as restoration of the suitable torque, when necessary).

4.4.4. The maintenance of cuff insulation seal.

4.4.5. The maintenance of lightning protection.

4.4.6. The maintenance of protection lights system of cuff.

4.4.7. The cleaning of cuff. After cleaning, the antenna cuff must be free of visually visible dirt, dirt and other stains must be removed.

4.5. In cases, when during the maintenance works it is determined that repair works are necessary:

a) it shall be stated in the document of the performed maintenance works signed by the representatives of both the Service Provider and the Purchaser;

b) the repair works required (according to the Order) shall be performed not later than in 60 days after the Order of the Buyer is submitted.

4.6. While performing the cuff maintenance and repair works, the respectful radar must be turned off. Therefore, the cuff maintenance Service Provider must ensure the performance of works within the shortest period of time. In any case all the necessary maintenance works (including the cuff cleaning) must be performed on every radar for no longer than three days.

4.7. In 10 business days after the moment the Contract comes into force the Service Provider shall be liable to make preliminary arrangements with the Purchaser regarding the date of routine maintenance of antenna cuffs of the radar.

4.8.. The Order for the maintenance and cleaning of the particular radar‘s antenna cuff shall be approved at least 30 days before the day of actual commencement of works.

- SERVICE PROVISION LOCATION

5. Service provision locations:

a) Vilnius radar,

b) Kaunas radar,

c) Palanga radar.

- WARRANTY OBLIGATIONS

6.1. A warranty of at least 3 months must be awarded to the provided Services. The term of the warranty shall be calculated from the moment of transfer of the provided Services to the Purchaser, i.e. from the moment when the deed on transfer – acceptance is signed.

6.2. The Service Provider shall undertake the liability to eliminate all the deficiencies of the Services identified during the warranty validity term using his own resources and at his own expense within 30 calendar days after the day on which the notification of the Purchaser on the deficiencies is sent, except for the cases when the deficiencies occurred due to the fault of the Service Provider.

- DOCUMENTS POVIDED DURING EXECUTION OF THE CONTRACT

7. The Service Provider is liable to record the performed antenna cuff maintenance works respectively. After performed maintenance of every radar antenna cuffs the recording documents must be signed by the representatives of both the Service Provider and the Purchaser. Not later than within one month after the document of the performed maintenance works is signed the Supplier is liable to provide the final report on the provided Services to the Purchaser, which must indicate the provided services (brief description), the achieved and/ or expected result, possible risks, conclusions, recommendations and etc.).