Annex 7.1 to Contract

TECHNICAL SPECIFICATION

FOR PART I OF THE OBJECT OF PROCUREMENT

- General requirements.

  - The procurement shall consist of 9 Security video surveillance servers  (hereinafter - the Goods) with the required parameters specified in Table 1.

  - The Goods must be provided new and unused, in the manufacturer’s original packaging.

  - The Goods must be covered by the manufacturer’s warranty for at least 5 years from the date of signature of the transfer-acceptance act.

  - All Goods must be brand new, unused and delivered in their original factory packaging. Factory refurbished “renewed”, “refurbished”, “remarked” components are not allowed.

  - The hardware components of the servers (processor, RAM, disks, adapters, power supply, etc.) must be fully assembled in the manufacturer’s factory and bear the computer manufacturer’s trademarks.

  - The Goods shall be delivered to the Buyer not later than 14 weeks after the date of entry into force of the Agreement at K. G. E. Manerheimo str. 8, LT-05131, Vilnius.

  - The Goods must be accompanied by detailed descriptions of technical parameters in Lithuanian and/or English.

When submitting a tender, the Supplier must fill in the column “Suggested value” in Annex 1 to the Technical Specification, indicating the specific size/value, function, performance or characteristic of the goods proposed, and in the column “Document (reference and/or page) attesting compliance with the requirement”, by ticking the page or other reference in the tender where the descriptions of goods or other technical documentation provided refer to the parameter, performance, function or feature in question, and which confirms the compliance of the proposed goods with the requirements of the technical specifications.

- Table 1. Technical requirements

| No. | Parameters | Minimum requirements |
| --- | --- | --- |
|  | Security video surveillance servers’ type | All servers shall be of the Rack type, mounted in a 19” wide rack, complete with sliding, fully retractable, brackets and all accessories required for the installation (rails, fixings, cable arm, cables). |
|  | Number of processors | No more than 1 |
|  | Processor architecture | x86 architecture processor supporting 64-bit operating systems and applications, virtualisation instructions at hardware level. |
|  | Cores in the server | At least 8 physical cores. |
|  | Processor performance requirements | The proposed processor must provide at least: - 69.7 units on the SPECint_rate_base2017 performance test; - 59.8 units on the SPECfp_rate_base2017 performance test. The results must be published on www.spec.org and included in the tender. The performance results provided must be measured on the proposed service station with the proposed processor. |
|  | RAM | No less than ECC DDR4-3200. Specify the model of RAM. |
|  | Installed general RAM | At least 64 GB. |
|  | Number of installed RAM modules | At least 2 |
|  | Drives | At least 2 SSD drives connected to RAID 1 controller: - SSD SATA 6G 3.84TB 2.5' (Solid State Drive), hot swap, hot plug; - Replaced defective drives must remain with the Buyer (Keep Your Drive). |
|  | Network interfaces | At least 2 1000 Base-T Ethernet ports. The network ports can be used to provide integrated server management and automatic firmware updates without the need for additional software or hardware. |
|  | External and internal input/output connections | At least 4 USB 3.2 GEN 1 minimum (of which at least 2 must be at the front and at least 2 must be at the back); - 1 special management LAN port that can switch the management traffic to any other integrated Gbit LAN port; - 1 monitor port. |
|  | Video adapter | Built-in video adapter with at least 1024 Mb memory |
|  | Power source. | hot swap, hot plug; at least 96% efficiency; suitable for 230 V 50Hz AC mains power. |
|  | Expandability of power sources | Option to add a second power supply or an uninterruptible power supply providing at least 4 minutes of battery life at a load of 280W. |
|  | Remote control adapter | There shall be a dedicated management controller, independent of the OS, with a dedicated 10/100/1000baseT Ethernet RJ-45 management network connection and the ability to connect via a USB/micro-USB connection at the front of the server. The Ethernet connection must be secured with a 128-bit key (SSL) or higher. The management controller shall support a remote management console with ODD. The management controller shall receive and record reports of abnormalities in the performance of the processor, memory, disk controller, disks and other parts of the server system. It shall support the sending of automatic notification of system failures by e-mail (SMTP), sending data to the monitoring system (SNMP V2 or higher), integration with Active Directory. The management controller shall support firmware updates for the server components. “KVM over IP” functionality shall be available (graphical interface independent of the operating system, virtual graphical console, management of virtual local CD-ROM devices). All licences necessary to ensure full remote control functionality of the server in perpetuity must be provided. The software shall be capable of centralised administration of all proposed servers. |
|  | Operating system | “Microsoft server 2022 essentials” or equivalent. |
|  | Compatibility | The server must be certified for the following operating systems: Windows Server 2019/2022 VMware vSphere™ 8.0 VMware vSphere™ 7.8 SUSE® Linux Enterprise Server 15 Red Hat® Enterprise Linux 8 |
|  | Manufacturer’s warranty | The equipment manufacturer must have at least 1 certified service centre in Lithuania. The supplier of the equipment must have the right to carry out after-sales service on the proposed equipment, or must have concluded an agreement with another company to carry out such after-sales service. The service station is covered by the manufacturer’s 5-year warranty during working hours (8x5), with a next business day (NBD) response time. Servicing of equipment must be carried out at the places of installation: Vilnius and Kaunas, with the address to be specified during the implementation of the agreement. Replacement of hard drives, memory and processor if there has been a “Prefailure Warranty” warning of their possible failure. Hard disks shall not be returned in case of failure and shall remain with the purchasing organisation. All of the above requirements must be guaranteed by the server manufacturer. It must be possible to check the level and validity of the warranty on the equipment manufacturer’s website. |
|  | Collection requirements | The proposed equipment must be new and not previously used. Renewed, refurbished, remarketed components are not allowed. All parts of the server to be assembled shall be assembled by the manufacturer of the server and marked with the manufacturer’s factory codes. |
|  | Part Numbers | A separate annex must contain the product codes (Part Number), a brief description and the quantities of all component parts. |