Prekių viešojo pirkimo-pardavimo sutarties

2024 m.                         d. Nr.

2 Priedas

PASIŪLYMAS

| Nr. | Pavadinimas | Kiekis | Mato vienetas | Kaina vnt. su PVM, Eur | Suma su PVM, Eur | Gamintojas, modelis |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Platforminis aviacinių krovinių keltuvas |  |  |  |  |  |
| 1.1. | Orlaivių vilkikai (mažos galios) | 4 | vnt. | 86 927,50 | 420 729,10 | Gamintojas Timsan (Turkija) modelis TIM 40 |
|  |  |  |  | Suma be PVM | 347 710,00 |  |
|  |  |  |  | PVM (21%) suma | 73 019,10 |  |
|  |  |  |  | Suma su PVM | 420 729,10 |  |

| Eil. Nr. | Reikalavimai | Tiekėjo pirkimui siūlomos prekės atitikties techninės specifikacijos reikalavimams patvirtinimas |
| --- | --- | --- |
| 1 | 2 | 3 |
|  | Driving speed: the maximum driving speed is not less than 25 km/h | CONFIRM Maximum driving speed 25 km/h |
|  | The tensile strength of the couplings is at least 35kN | CONFIRM – The tensile strength of the couplings 40 kN |
|  | Towing capacity: to tow aircraft or cargo trolleys with a total weight of at least 30 tons | CONFIRM 80 tons @ Dry Asphalt/Concrete and 1% Slope 55 tons @ Wet Concrete and 1% Slope |
|  | The total height of the tractor with the beacon must not exceed 250 cm, measured from the ground. | The total height of the tractor with the Air/Conditioning Unit 2465 mm |
|  | Tractor color: Green (RAL6031) | CONFIRM |
|  | The minimum weight of the tractor must be at least 6 tons. | CONFIRM – The weight of the tractor 7.5 tons |
|  | Coupling device - front and back at least 3 steps, the bottom step should be between 25 cm and 30 cm high, measured from the ground | CONFIRM – Coupling device - front and rear 3 steps, lower step 30 cm high |
|  | The cabin must be equipped with an air conditioning system that allows comfortable work in warm and cold seasons. | CONFIRM |
|  | The cabin has at least 2 seats | CONFIRM – Number of seats 2 |
|  | At least 1 orange beacon is installed on the cabin. | CONFIRM – Number of beacons 1 on the roof of the cab. |
|  | Engine power - at least 50 kW | Engine power – 74.9 kW |
|  | The engine must start without additional equipment or preparation when the air temperature is down to -10°C | CONFIRM |
|  | At temperatures lower than -10°C, starting must be facilitated by the installed additional engine heating equipment. | CONFIRM – Winter Kit will be provided. |
|  | Service brakes (pneumatic or hydraulic, disc) - for all four wheels | CONFIRM Front Brake: Dry Disc – Hydraulic Rear Brake: Wet Multi-Disc Hydraulic |
|  | Service brakes (pneumatic or hydraulic, disc) - for all four wheels | CONFIRM Front Brake: Dry Disc – Hydraulic Rear Brake: Wet Multi-Disc Hydraulic |
|  | Chassis: pneumatic tires power steering | CONFIRM |
|  | Electrical system 12V or 24V | CONFIRM – Electrical system 12 V |
|  | The product warranty is for at least 2 years. | Warranty 2 years |
|  | All notes and markings related to driving the tractor must be in Lithuanian or English | CONFIRM – The labels, decals and markings will be in English |

____________________________________